	SETUP FOR THE RC10L2 (www.rc10.com)

	Revelation Raceway INCLUDEPICTURE "C:\\Documents and Settings\\Null\\My Documents\\My Webs\\MYWEB15\\lgotemae.gif" \* MERGEFORMATINET 


Date: 12/8/02  Driver: Mark Anton   Track: 

	FRONT SUSPENSION
Upper arm mount: #2
[image: image1.png]



Caster shim position: #2
[image: image2.png]@




Front ride height spacers: kit
Steering block: STD
Camber, left –1 right –1/2
Toe-in: As near to 0 as possible
	FRONT SPRINGS
Springs: Assoc. .018
Kingpin: STD
Indicate shim position, qty and thickness:
[image: image3.png]



1: thickness: kit qty: 4
2: thickness: kit qty: 1
3: thickness: kit qty: 0

	SHOCK
Body: STD
Traxxas bladder inside shock
Oil 30 wt 
Spring silver

Limiter, inside 0 
outside 0
Piston # 1

Kit shock body and shock shaft length. 
	REAR SUSPENSION
axle height adjuster used: [image: image4.png]



T-bar thickness: .075"
Dampener lube: no
Dampener springs: STD #4517
Dan's Teflon Tape on upper plate
Roll stop insert: none
T-bar flex:  3 holes

	WHEELS & TIRES
	Front
	Rear
	Chassis: STD
Car body: Protoform Peugeot 905b MD
Wing/spoiler: side dams only
Motor: Reedy Ti 9x2
Pinion: 15      Spur gear: 84
Battery type: Orion Sanyo 3000HV
Indicate battery placement:
(Rearmost 3 holes on each side.)
Futaba 3PD radio, AIR737 servo, LRP Quantum Pro Speed Control.

	Tire type
	TRC

Pink
	Jaco Green
	

	Tire dia
	@2.30"
	@2.30"
	

	Stagger
	n/a
	n/a
	

	Car width
	standard
	standard
	

	Wheels
	TRC
	Jaco
	


 

	Rear axle: STD
Tire treatment type Zip Grip 2
	Shade in amount of tire treatment: (Full coverage)


	On Road Track Conditions:  Asphalt, clean, smooth, and high-bite for an outdoor track.  Unsprayed.  Track is 200’x100’ big. "Summer" layout.

	Race Comments: TQ'd and won.

	Car Comments: Will push for the first lap and go loose around the last lap. Otherwise very well balanced and fast setup for Pro10 mod. on this 1/8th scale gas oriented track.  This setup is specifically to use Pink front tires with Green or Aqua rears.


