	SETUP FOR THE RC10L2 (www.rc10.com)

	So Cal Raceway INCLUDEPICTURE "C:\\Documents and Settings\\Null\\My Documents\\My Webs\\MYWEB15\\lgotemae.gif" \* MERGEFORMATINET 


Date: 12/15/02  Driver: Mark Anton   Track: , 

	FRONT SUSPENSION
Upper arm mount: #2
[image: image1.png]



Caster shim position: #2
[image: image2.png]@




Front ride height spacers: kit
Steering block: STD
Camber, left –1 right -1/2
Toe-in: Very slight in
	FRONT SPRINGS
Springs: Assoc. .018
Kingpin: STD
Indicate shim position, qty and thickness:
[image: image3.png]



1: thickness: kit qty: 4
2: thickness: kit qty: 1
3: thickness: kit qty: 0

	SHOCK
Body: STD
Traxxas bladder inside shock
Oil 30 wt 
Spring silver

Limiter, inside 0 
outside 0
Piston # 1
Kit shock body and shock shaft length. 
	REAR SUSPENSION
axle height adjuster used: [image: image4.png]



T-bar thickness: .075"
Dampener lube: no
Dampener springs: STD #4517
Dan's Teflon Tape on upper plate
Roll stop insert: none
T-bar flex:  3 holes

	WHEELS & TIRES
	Front
	Rear
	Chassis: STD
Car body: Hot Bodies Toyota
Wing/spoiler: side dams only
Motor: Trinity P2k
Pinion: 18      Spur gear: 84
Battery type: Orion Sanyo 3000HV
Indicate battery placement:
(Rearmost 3 holes on each side.)
Futaba 3PD radio, AIR737 servo, LRP Quantum Pro Speed Control.

	Tire type
	TRC

Pink
	Jaco green
	

	Tire dia
	2.20"
	2.20"
	

	Stagger
	n/a
	n/a
	

	Car width
	standard
	standard
	

	Wheels
	TRC
	Jaco
	


	Rear axle: STD
Tire treatment type Zip Grip 2
	Shade in amount of tire treatment: (Full coverage)


	On Road Track Conditions:  Asphalt, cleaned but not sprayed. Moderately high-bite, indoors. Glass smooth except for the transitions from the banked oval to the infield.

	Race Comments: Won 2002 SoCal Grand Prix.

	Car Comments: Alternate to other SoCal setup. Significantly lower tire wear in an easy to drive, not nearly as aggressive car.  Can be slower for some people but it certainly is cheaper to use.


