[image: image1.jpg]mcsion



SST Mission Set Up Sheet

Driver;
Chris Kerswell
Circuit;
 Barham (A1 Layout)
[image: image2.jpg]


                           Meeting; BRCA Stock National Rd 2
                           Date;
11th May 2003

   [image: image3.jpg]


   

          Carbon/Carbon Composite   













Notes & Circuit Conditions;


Dry and hot, grippy


Qualified 2nd and won A-final.


Best time 18 / 311.300





Front Suspension;





Anti Roll Bar;	      Y/N thin/thick			Hub Carrier; Plastic/Alloy


Shock Position;   1-2-3-4-5 			On Wishbone;         1-2


Toe In;		0  		 deg.   	Inboard toe out;	Y/N


Castor;		7.25 deg. (1 front/3rear)	Lower Wishbone; Forward/Rear


Camber;		-1.5		 deg.		Upper Susp. Mount Position; 1-2


Ride Height;		5.5	 	 mm.		


Shock Oil;		70wt				Shock Length;	64	mm.


Shock Piston;	3 Hole			Droop;		1	mm.


Spring;		Red				Hub Carrier position; 1-2


Centre Track Rod Length;	42.5 mm	Steering arm position; 1-2		


				





Rear Suspension;





Anti Roll Bar;	      Y/N thin/thick			Hub Carrier; Plastic/Alloy


Shock Position;   1-2-3-4-5			On Wishbone;       1-2                                        Toe In;		2		deg.		Inboard toe in;       0-2	deg.


Camber;		-1.5		deg.	           Lower Wishbone; Forw/Mid/Rear


Anti Squat;		0				Upper Susp. Mount Position; 1-2


Ride Height;		6.5	           mm.		Hub Carrier position; 1-2 (1=in-line)


Shock Oil;		70wt		  		Shock Length;	64	mm.


Shock Piston;	3 Hole			Droop;		3	mm.


Spring;		Blue				Roll Centre; 	           1mm spacers





Pinion;		26	teeth


Spur Gear;	  	84	teeth


Pulley Size;   17t/20t


Front One Way; Y/N  


Front D/Shaft; Metal


Rear D/Shaft;  Plastic





Motor;	Fantom Silver Stock


Speedo;	LRP Quantum


Cells;		Orion 3000


Receiver;	KO 297


Servo;		KO 2143


Body;		Protoform Vectra





Front Tyre;	Pre-Glued Take Off CS-27	Front Insert;  Take Off Medium


Rear Tyre;	Pre-Glued Take Off CS-27	Rear Insert;  Take Off Medium








Notes; U2488 Stiff top deck and Normal top deck. Grey “stock” front belt. U2568 & U2570 RW pulleys.  Used LRP Blue Factor additive.  








		


		 











