[image: image1.jpg]mcsion



SST Mission Set Up Sheet

Driver;
Andy Wallis
Circuit;
 Worksop
Meeting; Worksop Winter series
Date;
18/11/01

   [image: image3.jpg]


   

          Carbon/Carbon Composite   












[image: image2.jpg]



Notes & Circuit Conditions;





Low traction





Front Suspension;





Anti Roll Bar;	      Y/N thin/thick					


Shock Position;   1-2-3-4-5 			On Wishbone;         1-2


Toe In;		0  		 deg.   	Inboard toe out;	Y/N


Castor;		5   		 deg.		Lower Wishbone; Forward/Rear


Camber;		2		 deg.		Upper Susp. Mount Position; 1-2


Ride Height;		4.5	 	 mm.		


Shock Oil;		35				Shock Length;	66	mm.


Shock Piston;	3				Droop;		 9	mm.


Spring;		Blue				Hub Carrier position; 1-2


Centre Track Rod Length;	Std.		Steering arm position; 1-2		


				





Rear Suspension;





Anti Roll Bar;	      Y/N thin/thick			


Shock Position;   1-2-3-4-5			On Wishbone;       1-2                                        Toe In;		2.5		deg.		Inboard toe in;       0-2	deg.


Camber;		2		deg.	           Lower Wishbone; Forw/Mid/Rear


Anti Squat;		0				Upper Susp. Mount Position; 1-2


Ride Height;		5	           mm.		Hub Carrier position; 1-2 (1=in-line)


Shock Oil;		35		  		Shock Length;	66	mm.


Shock Piston;	3				Droop;		11	mm.


Spring;		Blue				 








Pinion;			teeth


Spur Gear;	  86		teeth


Pulley Size;   17t/20t


Front One Way; Y/N  


Front D/Shaft;  Steel


Rear D/Shaft;   Steel





Motor;	Orion 12x1


Speedo;	LRP Quantum


Cells;		Schumacher XVS SMH


Receiver;	LRP


Servo;		KO 2015


Body;		Alfa 156





Front Tyre;	U6622 Sch. Yellow		Front Insert;  U6626 Soft


Rear Tyre;	U6622 Sch. Yellow		Rear Insert;   U6626 Soft








