	2WDRC.com

4WDRC.com
	X-6² SETUP SHEET
	X-FACTORY R/C

XFactoryRC.com

	Driver: Paul Sinclair
	Date:

	Track/Location: 2009 Starting Setup
	Event:

	 □Tight/   □Open
	   □Wet/  □Dry
	   □Smooth/  □Rough
	Slippery □  □  □  □  □ High-Bite

	FRONT

	SUSPENSION

RIDE HEIGHT:_ 22mm measured just behind bumper
CAMBER: __-2__°    TOE:__2__°     ■IN/     □OUT

CAMBER LINK:  INSIDE__1___     WASHERS__3___   OUTSIDE__A__
                                    1=inside                                                                                    A=inside

CASTER BLOCK:    □20°         □25°(stock)         ■30°

                           □BACK         ■FORWARD (stock)

BUMP STEER SPACERS:    ■.050” (stock)/    □OTHER:__2___

Spacers under outer tie-rod ballstud: 0

AXLE:    □TRAILING(stock)/   ■INLINE

AXLE HEIGHT:    □TOP         □MIDDLE         ■BOTTOM

STEERING ACKERMAN:   ■FORWARD(stock)         □BACK

SERVO POSITION: Forward

	SHOCKS

□ASSOCIATED/    ■LOSI

MOUTING(T/B):__1__ /__1__
                       1=inside

SPRING:_Orange_________
PISTON:__Red (56)_________
      ■STANDARD/      □DRILLED

OIL:30 wt (cold) or 35 wt (hot)
LIMITING(T/B): add limiting inside to get the ~73mm length (screw to screw) at full droop

	REAR

	SUSPENSION

TRANSMISSION:    ■4 Gear(Stock)/    □3 Gear 
TRANSMISSION HEIGHT:    ■.060(stock)/    □OTHER:______
RIDE HEIGHT:_ 22mm at back edge of chassis____
CAMBER:_-2_ °     TOE BAR:  □3°   ■4°     ANTISQUAT:1 Big, 1 Small

Extra Hub Toe In/Out: No

Rear Roll Bar: No
CAMBER LINK:  INSIDE:__2___  WASHERS:__0___  OUTSIDE:__C___
                                                            1=inside                                                                                 A=inside
HUB SPACING:    ■FORWARD         □MIDDLE         □BACK

ARM SPACING:    ■FORWARD         □MIDDLE         □BACK      
	SHOCKS

□ASSOCIATED/    ■LOSI

MOUTING(T/B):__3__ /__3__
                       1=inside

SPRING:___________________
PISTON:_ Red (56)__________
       ■STANDARD/     □DRILLED

OIL: 25 wt (cold) or 30 wt (hot)
LIMITING(T/B): 92 mm long (screw to screw) at full droop


	OTHER

	Alum Front bulkhead + 20 grams of weight. Front shocks 72.7mm long, screw to screw at full droop. Rear shocks 92.0mm long screw to screw at full droop. Rear arms .030" back for clearance. As the bite comes up, adjust together in .030" increments: tranny down, hubs back, and less antisquat. If bite comes way up, go to #2 shock hole on the rear tower and hubs all the way back.


	FRONT TIRES:
	FRONT INSERTS:

	REAR TIRES:
	REAR INSERTS:

	ESC:
	MOTOR: 6.5

	BATTERY:
	PINION/SPUR:_21_ / _81_

	WING: JConcepts Wide V / Front wing


