	2WDRC.com

4WDRC.com
	X-6² SETUP SHEET
	X-FACTORY R/C

XFactoryRC.com

	Driver: Sébastien Wartelle
	Date: 14-15.03.2009

	Track/Location: Loos
	Event: French National Rd1

	xTight / □Open
	□Wet / □Dry
	xSmooth / □Rough
	Slippery □  □  x  □  □ High-Bite

	FRONT

	SUSPENSION

Ride Height:_ _______________________________________
Camber: _-1____°    Toe:_1____°     □In/     □Out

Camber Link:  inside __1___     Washers_     __   Outside_A___
                                 1=inside                                                                          A=inside

Caster Block:    □20°           x 25°(stock)         □30°

                       □BACK        x FORWARD (stock)

Bump steer spacers:    □.050” (stock)/    xOTHER:_4_____

Spacers under outer tie-rod ballstud: ______
Axle:    xTrailing (Stock) / □Inline

Axle height:    xTop         □Middle         □Bottom

Steering Ackerman:   xForward(Stock)         □Back

Servo Position: ________

	SHOCKS



□Associated / xLosi

Mouting (T/B):__2___ /__1__
                       1=inside

Spring:_Green Losi___
Piston:_Red____
      xStandard/      □Drilled

Oil: _30 wt_____________
Limiting (T/B): _0_ / _0_

	REAR

	SUSPENSION

Transmission: x4 Gear (Stock) / □3 Gear 
Transmission Height: □.060 (stock) / xOther:_0____
Ride Height: _____________________________
Camber:_  _ °     Toe Bar:  x 3° / □4°     Antisquat:_1mm__
Extra Hub Toe In/Out: _____
Rear Roll Bar: _____
Camber Link:  Inside:_        _  Washers:__0___  Outside:__A___
                                                      1=inside                                                                           A=inside
Hub spacing:    □Forward         □Middle         xBack

Arm spacing:    □Forward         □Middle         xBack      
	SHOCKS

□Associated / xlosi

Mouting (T/B):_1___ / _3___
                                 1=inside

Spring:_ Red ___
Piston:_Red ____
       xStandard/     □Drilled

Oil: _32.5 wt_____________
Limiting (T/B): 3mm_ / ______

	OTHER

	Total weight 1660 grs

160grs in front battery

Q: TQ - F: 1st

	Front tires: Staggered
	Front inserts: 

	Rear tires: Mini Pin Jaunes
	Rear inserts: Blue

	Esc: Speed Passion
	Motor: 7.5

	Battery: GM 4000 Lipo
	Pinion/spur: 23 / 78

	Wing: Stock


