	[image: image1.jpg]


X – 6 Set-Up Sheet

	Driver: Arjan van de Graaf (aka Janus)
	Date: 

	Track: Base Dirt     

	Tight/Open

1=very tight 5=very open

1      FORMDROPDOWN 

	Wet/Dry

1=very wet 5=very dry

3      FORMDROPDOWN 

	Smooth/Rough

1=very smooth 5=very rough

5      FORMDROPDOWN 

	Slippery/High-Bite

1=very slick 5=very high bite

3      FORMDROPDOWN 


	Front

	Suspension
	Shocks

Losi  FORMCHECKBOX 
/Associated X

Oil Make: AE

Mounting (Top/Bottom):2   /   I

                                       1=inside                           

Spring: Silver 
Piston: 2 Std/Drilled - countersunk
Oil: 30 wt

Limiting (In/Out): 1   /   0

                   

	Ride Height: arms level ( 10 mm spacers + b44 cup)

Camber: 1o
Toe In/Out: Out      1,5o 
	

	Camber Link:  Inside: 2   Washers: 4 (thin)  Outside: B (GT-2) 



            1=inside



      A=inside
	

	Caster Block (20°/25°/30°): 30°

Castor Block Position (Forward/Back): FWD
	

	Bump Steer Spacers 0.05” (Stock 2)/Other: 1

Spacers under inner Tie-Rod Ballstud: 0
	

	Axle Trailing (Trailing/Inline) : Inline 
Axle Height Top/Middle/Bottom: MID

Steering Ackerman Forward (stock)/Back: Fwd
	

	Rear

	Suspension
	Shocks

Losi  FORMCHECKBOX 
/Associated X

Oil Make: AE

Mounting (Top/Bottom):1    /   I

                                       1=inside                           

* Cf Tower

Spring: silver

Piston: 2 Std/Drilled - countersunk
Oil: 30 wt

Limiting (In/Out): 0   /  0



	Transmission Height (Multiples of 0.3”): 0.000

Ride Height: axles  level ( 8mm aan spacers )

Camber: 1,5o       Toe Bar 3o/4o:  4  Antisquat: no.2

Extra Hub Toe In/Out:  B44 hub
	

	Camber Link:  Inside:  2    Outside:   B



            1=inside                           A=inside

Washers Ballstud:  Inside: 5 (thin) ->~2.5 normal shims  Outside: x
	

	Hub Spacing Forward/Middle/Back: FWD

Arm Spacing Forward/Middle/Back: BACK

Anti-Roll Bar: Nope
	

	Other

	Notes/rants/excuses:          - Esc setup (Venom 120a (Speedpassion clone)): 1-3-4-7-4-1-1-6-1-1 – max brake 80%
                                            - Losi plastic universals
                                            - 25 deg for more steering midcorner

                                            - adjust ride height for balance adjust
                                            - adjust lead for grip / reaction speed difference

                                            - for tracks with decent grip try 1 / I for front shock pos and 2 / C for front camber link

                                            - for more steering try 1 / M rear shock setting, or 3 deg rear toe.

	Front Tire:  

	Rear Tire:  

	Weights:   18G per cel, 65 in Nose (LIPO)

	Motor: 
	Pinion/Spur:         4 GEAR TRANNY

	Wing: mounted to front ½ gurney, Losi front wing on body








