	X – 6 Set-Up Sheet

	Driver: Arjan van de Graaf (aka Janus)
	Date: 

	Track: MAC Vlijmen BNK

	Tight/Open

1=very tight 5=very open

3      FORMDROPDOWN 

	Wet/Dry

1=very wet 5=very dry

5      FORMDROPDOWN 

	Smooth/Rough

1=very smooth 5=very rough

2      FORMDROPDOWN 

	Slippery/High-Bite

1=very slick 5=very high bite

2      FORMDROPDOWN 


	Front

	Suspension
	Shocks

Losi  FORMCHECKBOX 
/Associated X

Oil Make: AE

Mounting (Top/Bottom):1   /   I

                                       1=inside                           

Spring: Silver 
Piston: 2 Std/Drilled - countersunk
Oil: 30 wt

Limiting (In/Out): 1   /   0

Total shock length O: 73mm  / C 56mm

O-rings: 801

	Ride Height: arms level  (10,8mm shims, b44 cup)

Camber: 1o
Toe In/Out: Out      1,5o 
	

	Camber Link:  Inside: 2   Washers: 2   Outside: C (GT-2) 



            1=inside



      A=inside
	

	Caster Block (20°/25°/30°): 25°

Castor Block Position (Forward/Back): FWD
	

	Bump Steer Spacers 0.05” (Stock 2)/Other: 1

Spacers under inner Tie-Rod Ballstud: 0
	

	Axle Trailing (Trailing/Inline) : Inline 
Axle Height Top/Middle/Bottom: MID

Steering Ackerman Forward (stock)/Back: Fwd
	

	Rear

	Suspension
	Shocks

Losi  FORMCHECKBOX 
/Associated X

Oil Make: AE

Mounting (Top/Bottom):2    /   I

                                       1=inside                           

* Cf Tower

Spring: silver

Piston: 2 Std/Drilled - countersunk
Oil: 30 wt

Limiting (In/Out): 2   /  2

Total shock length O: 89mm  / C 67,5mm

O-rings: 801

	Transmission Height (Multiples of 0.3”): 0.060

Ride Height: 1mm higher than front (14,0 mm shims)

Camber: 2o       Toe Bar 3o/4o:  4  Antisquat: FULL no shims

Extra Hub Toe In/Out:  B44 hub
	

	[image: image1.jpg]


Camber Link:  Inside:  2    Outside:   B



            1=inside                           A=inside

Washers Ballstud:  Inside: 5 (thin) ->~2.5 normal shims  Outside: x
	

	Hub Spacing Forward/Middle/Back: FWD

Arm Spacing Forward/Middle/Back: MID

Anti-Roll Bar: Nope
	

	Other

	Notes/rants/excuses:          - Esc setup (Venom 120a (Speedpassion clone)): 1-3-4-7-4-1-1-6-1-1 – max brake 80%
                                            - Losi plastic universals conversion
                                            - Changed to 25 deg for more steering midcorner

                                            - track was brushed after every run, NO DUST

                                            - more rear camber than usual for more stability in fast corners

                                            - Result Q4 – P3

	Front Tire:  Losi Taperpin Red, FULL REAR insert

	Rear Tire:  Bigshot Silver with full original inserts

	Weights:   20 in celslots 2 and  5, lead under esc (40g) and servo (65g), additional 40 behind servo (LIPO)

	Motor: 6.5 bonded 
	Pinion/Spur:   22 / 78      4 GEAR TRANNY

	Wing: mounted to front full gurney, Losi front wing on body


· Losi Plastic universals conversion: Made Losi universals in the same way the B44 were made (http://2wdrc.com/tech/ModifiedCVA.htm ), with this you do need to use the old arms and shocktower to make room for the bigger universals

· The taperpins had much more reaction when used with a full rear insert

· Went back to the 2-B rear camber setting instead of the 2-C I’d been running recently to get more reaction out of the car

· Full antisquad was needed to get more on power steering, to get back some acceleration the tranny was placed at 0.060”

· Track was brused before every run, so a clean track for everyone ( happy happy, joy joy )

· Elvo said it was the car was to hostile, so it was spot on ;-)







